National University of Computer and Emerging Sciences

Lab Manual

Computer Organization and Assembly Language

Lab 10

Instructor 	Wajahat Ali Khan

Class 	lab 8, lab 4

Semester 	Fall 2022

Fast School of Computing

FAST-NU, Lahore, Pakistan

# Objectives

- Subroutines

- Display Memory

- String Instructions

Contents

Objectives	2

ACTIVITY 1:         [50 Marks]	2

ACTIVITY 2:         [50 Marks]	2

REFERENCES	4

Note for all questions: You can make as many memory variables, subroutines as you need. Must read all the manual before starting.

## ACTIVITY 1:  	 	 	 	 	 	 	[50 Marks]

Write a program that prints and prints the following messages: one message per keypress.

'msg1: Hi! I am YourName.'

'msg2: I am YourMode(Happy, Sad, etc).'

'msg3: I Study at FAST.'

'msg4: My Roll No is YourRoll#.'

Expected output after 4 key presses

## ACTIVITY 2:  	 	 	 	 	 	 	[50 Marks]

[org 0x0100] jmp code - equ 70 ; width offset - equ 50 ; starting x coordinate of line y equ 100 ; starting y coordinate of line c equ 60 ; color code: mov ah, 0 mov al, 13h int 10h ; draw diagonal 11: mov cx, x mov dx, y mov al, c u1: inc dx mov ah, 0ch ; put pixel int 10h inc cx cmp cx, x+w jbe u1 ;wait for keypress mov ah,00 int 16h mov ax, 0x4c00 int 21h

Given:

Start with given (𝑥, 𝑦) = (50,100) and in every loop iteration increment both Cx and Dx to reach (𝑥 + 𝑤, 𝑦 + 𝑤) = (120,170).

⇔

Required:

Similarly, you need to draw 4 lines as above satisfying the coordinates and increment or decrement as per requirement of the line as shown below.

⇔

## REFERENCES

- https://www.youtube.com/watch?v=ylmCcDf3Oek

- http://www.dosbox.com/download.php?main=1

- http://sourceforge.net/projects/nasm

- http://www.nasm.us/

- http://www.programmersheaven.com/download/21643/download.aspx (AFD)