National University of Computer and Emerging Sciences

Lab Manual

Computer Organization and Assembly Language

Lab 04

| Instructor | Wajahat Ali |
| --- | --- |
| Class | Lab 4 |
| Sections | B, C |
| Semester | Fall 2022 |

Fast School of Computing

FAST-NU, Lahore, Pakistan

# Objectives

- How to interpret the different types of jumps

- How to use the different types of registers and how to manipulate them in assembly language

- How to perform arithmetic operations with registers and conditional jumps

- How to use the debugger for viewing the available registers and their function

Contents

Objectives	2

ACTIVITY 1:	2

ACTIVITY 2:	2

ACTIVITY 3	3

ACTIVITY 4:	3

ACTIVITY 5:	3

REFERENCES	3

Note for all questions: You can make as many memory variables as you need

## ACTIVITY 1:

Initialize 𝑨𝑿 with last 4 digits of your roll number (for example, if your roll number is 16L-1105 then 𝑨𝑿 should be initialized with 1105).

Once initialized, write a program to swap every pair of bits in the AX register as shown in Table below:

| AX | Contents of AX (Your Roll #) |  |  |  |
| --- | --- | --- | --- | --- |
| Before | 0000 | 0100 | 0101 | 0001 |
| After | 0000 | 1000 | 1010 | 0010 |

## ACTIVITY 2:

Modify your program in Activity 1 to swap two bits as shown in Table below:

| AX | Contents of AX (Your Roll #) |  |  |  |
| --- | --- | --- | --- | --- |
| Before | 0000 | 0100 | 0101 | 0001 |
| After | 0000 | 0001 | 0101 | 0100 |

## ACTIVITY 3

Modify your program in Activity 1 & 2 to swap two nibbles as shown in Table below:

| AX | Contents of AX (Your Roll #) |  |  |  |
| --- | --- | --- | --- | --- |
| Before | 0000 | 0100 | 0101 | 0001 |
| After | 0100 | 0000 | 0001 | 0101 |

## ACTIVITY 4:

Initialize 𝑨𝑿 with last 4 digits of your roll number (for example, if your roll number is 16L-1105 then 𝑨𝑿 should be initialized with 1105). Store ̅𝑨𝑿̅̅̅ in 𝑩𝑿. Make a memory variable 𝑓, initialize it with 0 and compute

𝑓 = (𝐴||𝐵)&&(𝐴 ⊙ 0𝑥1𝐵𝐶𝐷)

|| is bitwise OR operation, && is bitwise AND operation whereas ⊙ is bitwise XOR operation.

## ACTIVITY 5:

Initialize 𝑨𝑿 with last 4 digits of your roll number (for example, if your roll number is 16L-1105 then 𝑨𝑿 should be initialized with 1105). Store ̅𝑨𝑿̅̅̅ in 𝑩𝑿. Make a 32-bit memory variable 𝑓, initialize it with 0 and compute

𝑓 = (𝐴 × 𝐵) + {𝐴, 𝐵}

× is Multiplication operation, + is Addition operation whereas {𝑨, 𝑩} concatenates 16-bit A and B to form 32-bit number.

ACTIVITY 6:

Differentiate between Near, Far and Short Jumps. Write your own assembly language programs and demonstrate how these jumps have been taken.

## REFERENCES

- "http://www.dosbox.com/download.php?main=1

- http://sourceforge.net/projects/nasm

- http://www.nasm.us/

- http://www.programmersheaven.com/download/21643/download.aspx (AFD)