COAL  – Fall 2021

Lab Manual 1 - Introduction

Activity 1: Setup

Make a separate folder COAL and NASM in your machine for example “D:\COAL\NASM”. Visit the link given below. Download and install NASM, AFD and DOSBOX, according to the instructions, in your NASM folder.

http://wetolearn.blogspot.com/2013/09/setting-up-afd-nasm-and-dosbox-for-8086.html

After installations double click “DOSBox 0.74-2 Options.bat” file and at the end of the file paste following lines:

MOUNT C D://COAL//NASM C:

(We are mounting C drive to our folder where we have saved AFD and we will save our .asm file in this directory)

You may follow links given below for Video Tutorial of Setup:

Tutorial part 1: https://drive.google.com/file/d/1N3lWL8hsN0ZbhF3tlNwCWWwjJ_eHQqk6/view?usp=sharing

Tutorial part 2: https://drive.google.com/file/d/10p8qyaOVOwF5lDighrMKE-uNYQX-c3bL/view?usp=sharing

AFD: https://drive.google.com/file/d/1eXnD3JEwBelFiJT6iIk7gluudV2Fu_iX/view?usp=sharing

NASM: https://drive.google.com/file/d/1ZoeE2MxjNaK7DdJKCacYfAJyn006MI_F/view?usp=sharing

Dosbox: https://drive.google.com/file/d/1DnaDIk4RoGBFDP1y4Dr3q89xwM3gx1d1/view?usp=sharing

Activity 2: Running your First Program

Follow these step in order to run your first program:

- Copy/paste following code in notepad

; this is a comment. Comment starts with semicolon ; this program adds three numbers in registers [org 0x0100] ;we will see org directive later mov ax, 5 ; AX = 5 mov bx, 10 ; BX = 10 add ax, bx ; AX = AX + BX mov bx, 15 ; BX = 15 add ax, bx ; AX = AX + BX mov ax, 0x4c00 ;terminate the program int 0x21

- Save this file as “ex01.asm” in your NASM folder e.g. “D:\COAL\NASM”:

- Go to NASM installation directory ( e.g. “D:\COAL\NASM”). Double click nasmpath.bat (batch file) and type following command there. (Your .asm file and nasm should be in one folder)

nasm ex01.asm -o ex01.com -l ex01.lst

- Above command will assemble your code and create ex01.com and ex01.lst files. Open ex01.lst file in notepad.

  - What is opcode of instruction “mov ax, someConstant”

  - Verify the above opcode everywhere the instruction has been used.

  - What does “B80500” mean?

  - Verify the opcode of instruction “mov bx, someConstant” throughout the machine code.

  - What is the offset of first instruction?

  - Why are offsets of second and third instructions 3 and 6?

  - What should be the size of ex01.com file?

  - Right click ex01.com and verify its size.

- Open DOSBox (by double clicking dosbox.exe), following window will appear:

- Write following command and press enter.

Afd ex01.com

(Your AFD.exe should be in same directory where we have installed everything)

- Above command will open the debugger and load your ex01.com file in it.

  - What is the value of IP register? And what will be its effect?

  - Note the initial values of data registers

  - Press F1 and watch the values of data registers

Activity 3: Explore different functions available in debugger