LAB MANUAL #6

Objectives:

- To learn and understand the working of NAND gate and NOR gate

NOTE:

- Implement the questions on logic works where it is mentioned.

- Only perform the part (a) & part (b) in question 2 & 3 using ICs (codes are mentioned below). Remaining part & question 3 can be performed using ready-made blocks.

Introduction to NAND Gate

74LS00 IC contains four 2-input NAND gates. The function table and connection diagram for this IC are shown below:

Function Table                                                               Connection Diagram:

| Inputs |  | Output |
| --- | --- | --- |
| A | B | Y |
| L | L | H |
| L | H | H |
| H | L | H |
| H | H | L |

H= Logic High, L= Logic Low

Introduction to NOR Gate

74LS02 IC contains four 2-input NOR gates. The function table and connection diagram for this IC are shown below:

Function Table:                                                                Connection Diagram:

| Inputs |  | Output |
| --- | --- | --- |
| A | B | Y |
| L | L | H |
| L | H | L |
| H | L | L |
| H | H | L |

H= Logic High, L= Logic Low

Question #1:

Simplify the Product-Of-Sums Boolean expression below.

NOR and NAND Implementation on logic trainer:

Question#2:

Implement on Logic Works the following using only the NAND gates

- Z = A.B                   (b)  X = A+B		(c)  XNOR

Note: Implement only part (a) & (b) using ICs

Question#3:

Implement on Logic Works the following using only the NOR gates.

- Z = A.B                   (b)  X = A+B		(c)  XOR

Note: Implement only part (a) & (b) using ICs

Question # 4: For the Boolean function  F1(A,B,C,D)=m(2,4,12,14)  do the following:

- Find truth table

- Find minimal SOP expression for Boolean function F1 using K-map. Draw K-map.

- Draw the resultant expression obtained in part (b) and implement on Logic Works using only NAND gates.