# Lab Manual# 7

Objectives:

- To learn and understand how to design a multiple output combinational circuit using XOR and XNOR

- To learn and understand the working of different types of decoders

- To learn and understand how to design a multiple output combinational circuit using Decoders

Exclusive-OR & Exclusive-NOR gates:

The figure given below shows the symbol of Exclusive-OR (XOR) and Exclusive-NOR (XNOR) gates.

XNOR gate                                 	               XOR gate

Boolean expression of XNOR gate is AB+ AB and Boolean expression of XOR is AB+AB. Boolean expression of XNOR gate can be implemented using XOR gate as shown in figure below:

Function Table:

| Inputs |  | Output |
| --- | --- | --- |
| A | B | Y |
| L | L | L |
| L | H | H |
| H | L | H |
| H | H | L |

H= Logic High, L= Logic Low

Connection Diagram:

74LS86 IC will be used for implementation of XOR gate function. 74LS86 IC contains four 2-input XOR gates. The function table and connection diagram for this IC are shown below:

2-to-4 line decoders:

74LS139 IC contains two fully independent 2-to-4 line decoders with active low enables. The function table and connection diagram for this IC are shown below:

Function Table:

| Enable | Selection Inputs |  | Outputs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| G | B | A | Y0 | Y1 | Y2 | Y3 |
| H | X | X | H | H | H | H |
| L | L | L | L | H | H | H |
| L | L | H | H | L | H | H |
| L | H | L | H | H | L | H |
| L | H | H | H | H | H | L |

H= Logic High, L= Logic Low, X= Don’t Care

Connection Diagram:

3-to-8 line decoders:

74LS138 IC contains 3-to-8 line decoder. The function table and connection diagram for this IC are shown below:

Connection Diagram:

Lab Tasks:

Question 1:

2-bit parity Checker

Implement a circuit that receives 2-bit message and outputs Error (E=0) if its parity is ODD

(Implement it using XOR and XNOR gates)

Question 2:

Make a truth table and implement 2x4 Decoder with a low enable using 8 AND gates and 3 NOT gates.

Question 3:

Implement 3x8 decoder using two 2x4 decoders and NOT gate

Question 4:

Implement a doubler circuit. It takes a 3-bit input and multiplies it by three and gives a 5-bit output.

(Implement it using XOR and XNOR gates)