# Class Activity # 15

Only high level logic diagrams required for questions 1 & 2. Detailed Decoder circuit is not required. Only use the hardware provided in specification, any single extra gate/decoder not allowed.

Question# 1:

Using one 3 to 8 line decoder, implement the following function

G (A, B, C) = Σ m (0, 2, 4, 5)

F (A, B, C) = Σ m (0, 1, 2, 3, 7)

Question# 2:

A combinational circuit is defined by the following functions:

F1= x'.y' + x.y.z'

F2= x' +y

F3= x.y + x'.y'

Implement the circuit with one decoder and external NAND gates only.

(Hint: Fill the k-map of each function to find its min-terms.)

Note:

- Label inputs/outputs properly to gain marks.

- Show condensed Truth table to show working in Design problems.

- Mention MSB and LSB also in Design problems